Minsterley Flood Action Group: Report for February 2024 to the Parish Council Meeting 2403 to be held on 12/3/2024

During February 2024, 112mm of rainfall was recorded; 66% above the mean for this month of a year.  The monthly 2024 average recorded is 87.875 mm todate in comparison with74.521mm/month for 2023 and 53.33mm/ month for 2022.  There were 2 distinct wet periods during the month from 5th to the 8th and the 20th-22th of the month which again passed through without any significant flooding within the parish.

The Flood Action Plan has been published and will be scrutinised at the next MAM which will be held at 13:30hours at Minsterley Parish Hall on the 13 March 2024 attended by all the Statutory Agencies, and other interested parties; to establish the priorities for the Action Plan going forward.

Following investigations of the existing storm and groundwater drainage systems regarding the drainage to the rear (West) of The Grove; remains as reported in August 2023 the Drainage and Flood Risk Manager is now in possession of the MFAG budget figures for his consideration to find funding for the implementation of a new alleviation drain.  This location was again inspected by the writer in late February and the majority of the field remained extremely wet including to the north west boundary due to a blocked LLFA ditch following the continued adverse weather events during December, January and February. However water was not passing through the garden at The Coppins.

At The Little Minsterley culvert head wall, has been cleared to remove silts and debris left in the wetted channel and over the grill on two sides.  It appears that a pipe discharging into the watercourse through the Head Wall is allowing water from the watercourse to enter the drainage system bringing about the flooding of 1 Little Minsterley remains unresolved.   During recent storms flood water from this dysfunctional drain has almost flooded the property on a number of occasions during this winter without the intervention of resident who has time and again managed to move sufficient detritus off the grill to allow flood water to pass through the culvert; in addition to pumping water away from the property drive area.  The Highway Authority having been aware should check this grill after each and every storm otherwise they may be liable to be sued for negligence.

**The debris grill on the head wall of the culvert adjacent to Callow Crescent and Plox Green Road has been cleared to 50% of its area the remaining area is filled with silts, river stones and detritus which requires urgent clearing following the recent storms to ensure the culvert itself does not become blocked; should this watercourse overtop its banks it could threaten flooding of The Square, Station Road and the businesses located there, which occurred in the noughties.  The south bank to the watercourse remains open to the amenity land owned by Shropshire Council there is no barrier fence in this location.   Also the debris grill is in need of very urgent repair and the storm drainage pipe outfall is blocked which drains the Callow Crescent Estate to the east.

Further monitoring will continue South of the School, the writer again had the opportunity to walk the river bank south to assess the state of the river banks and potential blockages within the wetted channel, as reported previously and continue to liaise further with the landowners.

The erosion of the river bank and levee leading to the undermining of The Grove public footpath has continued, and the temporary safety barriers have been replaced, the tarmac footpath remains in very poor repair, and this issue remains an agenda item for the parish meeting.  The eastern levee is being eroded due to over topping by the river when in flood, which is leading more frequent flooding and consequent erosion of the levee damaging the footpath; further discussion with NFF is required and with other authorities including public footpaths.

MPC & FAG continue to monitor surface drainage evacuation at Perret Close to report back to the LPA. Further John Bellis & the FAG are concerned that the construction of the outfall to the Flood Relief Channel, is inadequate. FAG to follow up.

The accommodation bridge, to the north of The Grove footbridge, withstood the recent flooding events and the river has been running above normal winter levels since the passing of Storm Babet.  The owners are considering the removal of the old existing bridge structure at an appropriate time.

FAG will be working with the landowner to reinforce the existing east bank of the river to safe guard the accommodation bridge at The Grove, “the ordinary watercourse consent” was not granted because SC- FWM didnot receive payment, as previously reported.

At Brook Cottage the riverbank reinforcing work to the orchard as been washed out during the winter storms and the existing gabions will be relocated during the season when works can be completed within the wetted channel.

All other unresolved matters remain as stated in previous reports and particularly those listed in the February/March report 2022.

The writer previously had the opportunity to visit Wagbeach and was staggered by the damage inflicted by storm Babet in October 2023, discovering that the whole engineering project to the river bank stabilisation works around the culvert outfall at Wagbeach had to be reworked following Storm Babet, and has since returned to establish further facts regarding the issue of flooding and river damage.

Residents continue to be perturbed by the volumes water flowing through a tributary passing through the hamlet and these water volumes are causing great concerns to the residents due to the potential for damage wihin the drainage channel which is attached to their property above the notional damp proof membrane.  This situation was previously exacerbated when Highway Drainage engineers improved road surface water drainage off Drury Lane by diverting the water into the open land drainage ditches to the east particularly adjacent to Drury Lane Farm, which flow into the watercourses that flow through the middle of Wagbeach to join the Minsterley Brook.

There is also another landslip to the west verge of the A488 between Hope Mill and Hope Mill Bridge just south of the Minsterley Parish Boundary in the Parish of Worthen with Shelve, caused by river waters overtopping from Santley river ford into the A488 flowing north re-entering the river from the road at the location given causing the river bank and road embankment which is one and the same to collapse around an existing road drainage outfall which is formed using wire gabions which have also moved.

Footnote: In 2014 the SC Highway Authority commissioned a survey of the highway surface water drainage conducted by consultants Mouchel; from Shrewsbury to The More Arms level of the A488.  And found and concluded that the drainage provision was inadequate for whole 17mile section of highway.  Thus surface water from the highway is collected in an uncontrolled manner contributing to the flooding issues within the Minsterley and Hope Valleys.  The FAG and the SC ward councillor, Heather Kidd are hoping to combine to assist one another to action flood resilience actions to overcome these matters.

It has also been established at the Muller effluent plant outfall pipe that this flood was 335mm higher than the previous highest record which was in July 2012.

A report is still awaited from SC-Bridge Structures on the progress of the feasibility study for alleviation works around the Bridges in The Square, remains abd.  It should be noted that damage has occurred to the retaining wall forming the river bank on the east side supporting The Grove footpath adjacent to 4 Bridge Mews, terraced dwelling and the balustrade post no.6 north of The bridges.

**It is noted that the highway gully outside the Church Lych Gate is blocked abd, and went onto flood the front court yard and entrance to Rhodes Cottage attached to Shropshire Pressure Washers, causing some water damage internally.

**It is also noted that it is apparent that the line to the road gully outside Hawthorne House is broken or blocked because whenever it rains the front court yard to Rhodes Cottage floods and water is entering the front entrance lobby and sitting room causing water damage to the interior fittings eg carpets etc. These gullies and lines must be cleared and left operational; report to Fix My Street again.

** The road drainage outfall to the north of the Bridges in The Square on the west bank requires clearing of river stones and silts otherwise the highway outside the Church will become flooded.

The Grove Flood alleviation ditch remains clear of vegetation, the debris grill to the head wall also remains clear in readiness for the next flood, and the ditch has running water through it presently.

It has also been established that the eastern river bank to the south of The Square, Plox Green Road and between the Brooklands accommodation bridge has been raised during to 20th century (ie the last 100years) 300-400mm  including the highway itself, thus holding more river water within the wetted channel compromising those eastern banks that do not reach the same height allowing more property to be flooded north of the Church

The attenuation ponds have now been commenced on the Leigh Road, Laureate Ley development.

** = These are actions that could be taken presently to the work on by the FAG on and SC routine maintenance.

The Mfag report that our able colleague, Chairman Geoff Brown, continues recovering from his heart attack and would like to take this opportunity to wish him well for his continued recovery to good health and well being.

Originator: Cllr Doug Jones dated 08/3/2024; FAG Member