Minsterley Flood Action Group:

Report for March 2025 to the Parish Council Meeting 2504 to be held on the 8 April 2025

During March 2025, 11mm of rainfall was recorded, with last recorded rainfall on the 19 March, thus it is currently a drought period.  Total for the year todate = 142.5mm; the monthly 2025 average recorded todate is 47.5mm suggesting a very average year if this trend is to continue, in comparison with 81.52mm/month in 2024: 74.521mm/month for 2023 and 53.33mm/ month for 2022.

The next MAM will be held on the 30 April 2025 at Minsterley Fire Station, a new venue, following the fire service expressing an interest in becoming involved in the Flood alleviation and emergency procedures adopted to cope with flooding events and the subsequent clear up operations. Further joining the debrief following a flood event within the Minsterley Parish and adjoining areas within the Rea Catchment Area which includes the Minsterley Brook Catchment.

The agenda for the MAM has been published and can be found on the MFAG website, the content of which includes many of the issues included in the February 2025 monthly report.

At the Little Minsterley culvert head wall trash grill has been cleared, and remains clear.

It should be noted that the Trash Screen over the headwall to the Culvert under Plox Green Road at the Callow Crescent junction and Ash Lea Estate remains completely covered by detritus, there is now a flood waiting to happen. It is too dangerous for a volunteer to attempt to clear as there are deep silts in front of the screen.

SCH closed the A488 south of Plox Green to Hope between the hours of 09:30- 15:30 each day from 31 March to 4 April 2025 inclusive to enable emergency maintenance works to be undertaken including removing trees from the verge and river embankments together with conducting engineering assessments to establish how repairs might be executed to control erosion during flooding events.  The findings of those investigations are awaited with interest but hopefully solutions will be found to abate the destruction caused due to flood waters during flooding events.

The appointment of Flood Wardens is still awaited, this process is awaiting clearance from SC.

RIDB that a land drain that passes to the rear of The Grove properties, that has ‘blown’ to the rear of no.12, which leaks when the Malehurst flood plain is full  because the outfall is blocked.  The RIDB are in discussion with the EA with a view to removing silts at the confluence of the Minsterley Brook with Rea Brook at Malehurst to lower the Rea winter flow level to allow connecting water courses and drainage to outflow into the Rea Brook, awaiting a decision from the local SRT management Board.   This problem remained a constant but has dried up following the drought period being experienced at the moment.

It is imperative that The Grove flood alleviation works to the west MAM Agenda point 4.2 is implemented without further delay.

Following the announcement of additional funding including for the Rea Catchment on the 3April 2025 it is hoped that issues in and around Minsterley can be addressed to bring about flood resilience and the Minsterley FAG will be lobbying the local MP, Ms Julia Buckley in an endeavour to ensure funds are available.

Originator: Cllr Doug Jones dated 08/04/2025; FAG Member